Programming GPIO’s

# GPIO Driver Access

The WinCE demo image includes GPIO driver that allows direct access to GPIO functionality. To get familiar with GPIO’s hardware functionality you should read the

“CM-X270 Reference Guide” section 5.1 “General purpose I/O”.

The GPIO driver is a stream driver that can be accessed via “GIO” prefix.

To hide the implementation details the following interface is defined (in “gpio.h”) for GPIO’s access:

- HANDLE GPIOOpen()

Creates the GPIO device handle that is used in all other functions.

- VOID GPIOClose(HANDLE hDevice)

Deletes the GPIO device handle.

- BOOL GPIOIsAvailable(HANDLE hDevice, DWORD num)

A must call function before accessing any GPIO. This function checks whether the current GPIO is free to use as GPIO and is not used by other driver (for exmple keypad driver).

- BOOL GPIOSetAsOutput(HANDLE hDevice, DWORD num, GPIO_VALUE val)

Configures the specified GPIO number(num) to be an output and sets the default value (val).

- BOOL GPIOSetAsInput(HANDLE hDevice, DWORD num)

Configures the specified GPIO number(num) to be an input.

- BOOL GPIOGetValue(HANDLE hDevice, DWORD num, GPIO_VALUE *gpioVal)

Gets the specified GPIO number(num) state. The value can be GPIO_LOW(0) or GPIO_HIGH(1).

- BOOL GPIOSetValue(HANDLE hDevice, DWORD num, GPIO_VALUE val)

Sets the specified GPIO number(num) state. The value can be GPIO_LOW(0) or GPIO_HIGH(1).

- BOOL GPIORegisterInterrupt(HANDLE hDevice, GPIO_INTERRUPT interrupt)

Registers the specified GPIO number(num) to be a source of the interrupt that will be trigered at the specified edge (see the self explenatory GPIO_EDGE_SETTINGS).  As a result the handle for interrupt event for this GPIO is returned in “GPIO_INTERRUPT.Event” member.

- BOOL GPIOUnRegisterInterrupt(HANDLE hDevice, DWORD num)

Unregisters the specified GPIO number(num) to be a source of interrupt.

- void GPIOInterruptDone(HANDLE hDevice, DWORD num)

Should be invoked for the specified GPIO number(num) at the end of each interrupt processing.

# GPIO Sample Application

This GPIOSample application illustrates the most basic utilization of CM-X270 GPIO’s in Windows CE:

- How to configure GPIO pin on CM-X270 module as output and manipulate it.

- How to configure GPIO pin on CM-X270 as input and respond to changes in signal level.

- Basic polling technique.

- How to setup an event that will be triggered when the system detects an edge on an input pin.

The GPIOSample manipulate the following GPIO’s:

- GPIO 86 (pin 7 on SB-X270 P16) 	- will be set as output

- GPIO 107 (pin 45 on SB-X270 P16) 	- will be set as input

- GPIO 104 (pin 6 on SB-X270 P16) 	- will be set as input.

To test the results of GPIOSample make sure that you can read the voltage of GPIO86 (with DVM) and can control the state (possibly with switch or button) of GPIO107 and GPIO104.

The GPIOSample performs the following action with above GPIO’s:

- Configures an event (with GPIORegisterInterrupt)that will be signaled when any edge is detected on GPIO107 (it is also possible to detect only rising or falling edge).

- On the next step the program waits for the any edge on GPIO107.

During the wait GPIO86's value is inverted every 3 seconds. You can monitor this transition on that pin with DVM.

- At this point, you will have to trigger an edge on GPIO107 to continue. Notice how the program signals the interrupt is done (GPIOInterruptDone) and later clears the interrupt (with GPIOUnRegisterInterrupt).

- The next phase demonstrates polling. The program will continually check the value on GPIO 104, when it will be set as logical 1 (externally by you) the program is over.