Disk.bin Generator Manual

## 1	Introduction

A standard image file - NK.BIN - is generated by the Platform Builder. However, when a Flash Disk is available in the system (as is the case with the CM-X270), NK.BIN defines only the file system within its own range. In other words, a system based only on NK.BIN is not capable of using the rest of the Flash Disk as a filesystem. To overcome this limitation, the NK.BIN file is merged with a larger image structure that includes the surrounding filesystem.

### 1.1	About The Application

This application is used to create disk.bin files that can be written to the NAND flash of the CM-X270.

The disk.bin file consists of un-partitioned space including the nk.bin file and one or two unformatted partitions.

The disk.bin structure is arranged as shown in Table 1. The first time the system is booted up; WinCE is given the task of actually formatting the partitions with the FAT file system.

The program calculates the area boundaries so that they match the boundaries of the NAND flash block. Therefore, the corruption of a specific NAND flash block can impact only one area.

### 1.2	New Features

The basic structure of the disk generated by this program was modified in order to enable a failsafe update of the runtime image. The new disk layout is described in Table1.

The new layout's unpartitioned space now holds two WinCE images. This feature enables a Failsafe Update of the NK.bin using Compulab's "Image Updater Utility". Failsafe Update is a quick, simple and safe method to update your WinCE runtime image.

In order to provide backward compatibility, it is still possible to generate the old version of the disk layout. In order to do so, you simply need to check the "Create Legacy Disk" checkbox.

You will also notice the improved graphical user interface which makes the program much easier to understand and operate.

Table 1

This is the basic layout of the NAND flash.

## 2.	Operating the Program

The program default mode allows you the quick disk.bin generation that is very helpful during the evaluation.

Before preparing the production disk.bin the customer is advised to use the advanced mode for fine-tuning the exact flash layout.

### 2.1 	Disk.bin Generation Instructions

Figure1

The program’s basic GUI.

- Using the dropdown list box, select your NAND flash’s size (in MB).

- If you want to create a legacy disk.bin which is compatible with versions of ARMmon older than 1-May-2007, you can check the box "Create Legacy Disk".

- Using the "Browse" button, select the input nk.bin file.

- Click the Generate button and wait for the process to end. The newly generated disk.bin is located in the same path as the input nk.bin file.

### Disk.bin Generation Advanced Instructions

Figure2

The Advanced Setting dialog.

- In the basic dialog box, choose a disk size, input file and choose legacy or normal mode as described previously.

- Click on the "Advanced" button. The dialog shown in Figure2 will load up.

- Image max size: the maximum size of the nk.bin that can be stored in the unpartitioned space. You should pick the size most suited to your needs. A good rule of thumb is to start with the size of your current image and add a few more MB to allow for potential future growth.

- Un-partitioned space: should be big enough to hold two images (image size was specified in "image max size") and three more blocks (one for the partitions table and the other two are the image control blocks).

- If you wish to create a single partition, mark the corresponding checkbox.

- To create two partitions, leave the Checkbox empty, and choose the first partition’s desired size. Its size will be adjusted to coincide with the NAND block boundaries.

- Press “Calculate”; this will display the sector (512 bytes) numbers of the boundaries in disk.bin.

- If you wish to merge the nk.bin file with an existing disk.bin file, check the corresponding checkbox. Using the Browse button, select the disk.bin file.

- Press OK to save the settings.

- Click the Generate button and wait for the process to end. The newly generated disk.bin is located in the same path as the input nk.bin file. If you chose to merge the image with an existing disk.bin file, the modified disk.bin’s location is left unchanged.

## 3.	Backward Compatibility

When in Legacy mode, this version of the Disk.bin Generator is identical to the previous version.  This feature is especially important if you are running a version of ARMmon dated prior to 1-May-2007. If this is the case you must use Legacy mode, otherwise you won't be able to boot up the system using the generated disk.bin.

Table2 describes the basic layout of the disk generated in legacy mode.

Table2 – Legacy disk basic layout.

## 4.	Program Internals

The previous version of this program had some input parameters that no longer exist in the new version (they were dropped in order to simplify the program). This section explains how the program selects those values.

- Heads: 4 for NANDs below or equal 128MB, otherwise it's 16.

- Sectors on track: always 63.

- Block size: 16KB for NANDS below 128MB, 128KB otherwise.

- Sector size: always 512 bytes.

## 5.	Notes and Errata

- Sometimes it is impossible to merge an nk.bin file with a disk.bin because the image file selected is larger than the maximum image size supported by your disk. If this is the case, the application will inform you of that. In this case, you must generate a new disk.bin file.

- You can learn more about updating the WinCE runtime image in the “ImageUpdater manual.doc” under the “Docs” directory in the image archive.

- When you generate a file with a single partition, that partition will not cover the full disk size. To utilize the full disk space, allow the program to create an extended partition as well (the extended partition will cover the rest of the disk). Alternatively, you can create more partitions using the WinCE Storage Manager.

- The output disk.bin file size is determined by the sizes of the nk.bin, the unpartitioned space and the first partition size. If you want to cut down the size of the disk.bin, you should create a single partition. Choosing legacy mode will also reduce the size of the generated file.

- In order to allow some room for bad blocks, the disk size is set to a few MB less than the actual NAND size. The following table describes the selected disk size selected.

| NAND (MB) | Disk Size (kb) |
| --- | --- |
| 16 | 15424 |
| 32 | 30848 |
| 64 | 61696 |
| 128 | 123392 |
| 256 | 246784 |
| 512 | 493568 |