Image Updater Utility

# Introduction

This utility used to update the Win CE run time image stored in the NAND Flash of CM-X270.

The Image Updater is executed on the CM-X270 and can update the WinCE NAND image from any available media, such as network drive, USB mass storage, PCMCIA and SD Card.

There are 3 modes of the update:

- Failsafe NK.bin Update (the preferred mode)

- Legacy NK.bin Update (for backward compatibility)

- Disk.bin Update

Figure 1 - Image Updater GUI.

# Image Update instructions

- From Windows CE (WINCE) run "ImageUpdater.exe".

- In "Update Type", choose an update mode.

- If you want to skip the verification process, uncheck the corresponding box; otherwise, leave it checked.

- Using the Browse button, choose an input file.

- Click the "Update" button and wait for the process to end.

To access a remote computer, from the Open File window, enter the file name "\\[host computer IP address or DNS name]", as shown in Figure 2

Figure 2 – Browse Dialog.

## Failsafe NK.bin Update

This is the recommended mode. In this update mode, the utility safely updates the WINCE image (NK.bin). The rest of the disk is left unmodified.

This procedure is failsafe because it updates the system's inactive image (the basic layout of the disk is described in the following pages). Only after the entire file has been successfully written to the disk does the utility activate the inactive image. This method ensures that in the event of a serious failure during the update process (a power outage for example), the system's original runtime image remains unharmed.

In order to use the Failsafe Update feature, you will need to do the following:

- Ensure that your card has the ARMmon version supporting Failsafe partition layout.  (ARMmon with a build date later than 1-MAY-2007).

- Generate a failsafe aware disk.bin. (Use the disk_bin_generator released with the WINCE image archive starting from April 2007).

- Perform Disk.bin Update.

## Legacy NK.bin Update

Supported for backward compatibility with older images.

This mode updates the WINCE Image (NK.bin) in un-partitioned space. The data on file system partitions is left unmodified.

Legacy Update is provided for legacy NAND Flash layouts supporting only WINCE Images in their un-partitioned space.

## Disk.bin Update

A "Disk.bin" is a file generated by the “Disk.bin Generator” program provided with the WINCE runtime image from CompuLab. This file contains both the runtime image and the formatted partitions with data.

This update mode will update the entire NAND Flash. The partitions table will be overridden, so data stored in the NAND partitions will be lost.

Since the process will override the NAND Flash, the input disk.bin file can’t be stored on the NAND Flash. It can be stored remotely in a network location or locally in a USB, SD or PCMCIA storage device.

NOTE:

Before the update starts, you will be prompted to restart WINCE, click OK and the system will automatically reboot. This step of the program resets some registry values that will disable background flushing to the NAND Flash after the system boots up. Disabling background flushing is vital for the correct completion of this procedure.

# NAND Flash Layout

## NAND Flash New (recommended) Layout

| Logical Disk Layout | Usage |
| --- | --- |
| Block 0: | Partition Table |
| Block 1: | Image 1 Control block |
| Block 2: | Image 2 Control block |
| Unpartitioned Space | Image 1 |
|  | Image 2 |
| Partitioned Space | User File System |

At any given time only one image is active, blocks 1 and 2 contain all the relevant information about both images. This structure enables a Failsafe Update of the active runtime image by updating the inactive image and activating it only at the end of the update process.

## NAND Flash Legacy Layout

This is the basic recommended layout of the NAND Flash for images that do not support the Failsafe Update.

| Logical Disk Layout | Usage |
| --- | --- |
| Block 0: | Partition Table |
| Unpartitioned Space | Image 1 |
| Partitioned Space | User File System |

Only one image is stored in the unpartitioned space.

# Notes and Errata

- It is impossible to do a Failsafe Update on a Legacy NAND Flash layout. ImageUpdater automatically detects the NAND Flash layout and enables only those buttons appropriate for your system.

- Sometimes it is impossible to run the Failsafe Update of NK.bin because the image file selected is larger than the maximum image size supported by your NAND Flash layout. If this is the case, ImageUpdater will inform you and you will have to generate a new disk.bin file and run the “Disk.bin Update” procedure.

- Sometimes it is impossible to run the Legacy NK.bin Update because the disk's unpartitioned space isn't big enough. If this is the case, the application will inform you and you will have to generate a disk.bin file and run the “Disk.bin Update” procedure.